Analysis of SOLWEIG v2022a (UMEP QGIS plugin)

Week 1

What SOLWEIG is: SOlar and LongWave Environmental Irradiance Geometry, a raster, time-stepping radiation model. Given a 3-D urban surface (buildings, ground, vegetation) + weather, it computes the radiation reaching a person at every pixel and converts it to Mean Radiant Temperature (Tmrt), the dominant driver of outdoor heat stress.

- Spatial unit = one raster pixel (e.g. 1 m or 2 m grid cell).

- Time unit = one row of the meteorological file (one timestep).

- The model is therefore a loop over timesteps, each producing full maps + optional point time-series.

The equation - Per pixel and per timestep, SOLWEIG sums radiation from 6 directions (up, down, N, E, S, W) for shortwave (K) and longwave (L):

Sstr = absK · (Σ shortwave fluxes) + absL · (Σ longwave fluxes)

Tmrt = (Sstr / (absL · σ))^(1/4) − 273.2 [°C]

| Symbol | Meaning | Source |
| --- | --- | --- |
| Sstr | total radiant flux density absorbed by the human body | blending sun (shortwave, K) and heat (longwave, L) from all 6 directions |
| σ | Stefan–Boltzmann constant (5.67051e-8) | hard-coded |
| absK | human shortwave absorption | parameter (default 0.70) |
| absL | human longwave absorption | parameter (default 0.95) |
| K↓ K↑ Kside | shortwave fluxes | sun + shadows + albedo + SVF |
| L↓ L↑ Lside | longwave fluxes | air/surface temp + emissivity + SVF |

Two-layer architecture discovered:

| Layer | File(s) | Responsibility |
| --- | --- | --- |
| Orchestration / GUI | solweig.py, solweig_dialog.* | collect inputs, validate, set defaults, manage thread, write outputs |
| Physics core | Solweig_2022a_calc.py + SOLWEIGpython/* | one timestep of radiation physics → Tmrt + fluxes |

Hierarchical module map

Full file catalogue

Plugin / GUI layer (QGIS-facing) – for application-specific QGIS

| File | Role |
| --- | --- |
| __init__.py (UMEP root) | QGIS entry point; classFactory(iface) |
| metadata.txt (UMEP root) | Plugin manifest |
| UMEP.py | Master menu builder; |
| SOLWEIG/solweig.py | Plugin class: dialog, raster load/validate, params, threads start point |
| SOLWEIG/solweig_dialog.py | Loads the Qt Designer UI file – Desktop specific GUI |
| SOLWEIG/solweig_dialog_base.ui | Dialog layout + default parameter values. |
| SOLWEIG/WriteMetadataSOLWEIG.py | Writes RunInfoSOLWEIG_*.txt (settings record). |
| SOLWEIG/landcoverclasses_2016a.txt | LC code → surface thermal/optical properties. |
| SOLWEIG/tmrt.qml | QGIS style for the average-Tmrt raster. |

Compute layer (physics)

| File | Role |
| --- | --- |
| SOLWEIG/solweigworker.py | Worker (QThread): the timestep loop + output writing |
| SOLWEIGpython/Solweig_2022a_calc.py | Core model - one timestep: shadows, surface temp, 6-direction K & L, Tmrt. |
| SOLWEIGpython/Tgmaps_v1.py | per-pixel surface-temperature wave coefficients. |
| SOLWEIGpython/gvf_2018a.py | Ground View Factors (ground/walls radiating onto a pixel, per direction). |
| SOLWEIGpython/Kup_veg_2015a.py | Reflected/upward shortwave per direction. |
| SOLWEIGpython/Kside_veg_v2022a.py | Side shortwave. |
| SOLWEIGpython/Lside_veg_v2022a.py | Side longwave. |
| SOLWEIGpython/Lcyl_v2022a.py | Anisotropic longwave. |
| SOLWEIGpython/cylindric_wedge.py | Wall-shadow fraction seen from a pixel. |
| SOLWEIGpython/TsWaveDelay_2015a.py | Surface-temperature thermal inertia |
| SOLWEIGpython/daylen.py | Sunrise/sunset & day length. |
| SOLWEIG/PET_calculations.py | PET at POIs. |
| SOLWEIG/UTCI_calculations.py | UTCI at POIs. |

Shared utilities (Utilities/SEBESOLWEIGCommonFiles/)

| File | Role |
| --- | --- |
| Solweig_v2015_metdata_noload.py | Met parsing + sun position, DOY, daily max altitude, default leaf-on flag. |
| sun_position.py | Astronomical solar geometry. |
| clearnessindex_2013b.py | Sky clearness index CI (cloudiness proxy). |
| diffusefraction.py | Splits global radiation into direct + diffuse. |
| shadowingfunction_wallheight_13/23.py | Shadows (buildings / buildings + vegetation). |
| Perez_v3.py, create_patches.py | Anisotropic sky patches with relative luminance. |

Week2

INPUTS (full spec)

SOLWEIG has three input categories:

- spatial rasters,

- the meteorological file,

- scalar parameters

### Spatial raster inputs

Main rule: every raster must share the same extent, resolution, and pixel grid as the DSM, or the run aborts with an error dialog.

| # | Input | File type | Unit | Required? | Produced by | Feeds (model use) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DSM - ground + buildings | GeoTIFF | metres | Yes (always) | DSM Generator | shadows, building height, scale, lat/lon, altitude |
| 2 | CDSM -vegetation canopy | GeoTIFF | metres | Optional (veg scheme) | Tree Generator | vegetation shadows, transmissivity psi |
| 3 | TDSM -vegetation trunk zone | GeoTIFF | metres | Optional | Tree Generator or auto = CDSM × trunk-ratio | below-canopy geometry |
| 4 | DEM - bare ground | GeoTIFF | metres | Only if buildings not from land cover | DSM Generator | buildings = DSM − DEM |
| 5 | Land cover grid | GeoTIFF | class code 1–7 | Optional (LC scheme) | Land Cover Reclassifier | per-surface albedo/emissivity/Tg via Tgmaps_v1 |
| 6 | Wall height | GeoTIFF | metres | Yes | Wall Height & Aspect | wall longwave/shortwave geometry |
| 7 | Wall aspect | GeoTIFF | degrees 0–360 | Yes | Wall Height & Aspect | direction each wall faces |
| 8 | SVF set svfs.zip | zipped GeoTIFFs | fraction 0–1 | Yes | Sky View Factor Calculator | sky fraction per direction (2.3.2) |
| 9 | Shadow matrices shadowmats.npz | NumPy archive | - | Optional (anisotropic sky) | Sky View Factor Calculator | per-sky-patch shadowing (Perez) |
| 10 | POI point layer | vector (points) | - | Optional | user | locations for PET/UTCI time-series |

Contents of svfs.zip

| Group | Files | Meaning |
| --- | --- | --- |
| Total + cardinal | svf, svfN, svfS, svfE, svfW | sky fraction (building geometry) |
| Vegetation (sky-blocking) | svfveg, svfNveg, svfSveg, svfEveg, svfWveg | canopy blocking sky |
| Vegetation (building-blocking) | svfaveg, svfNaveg, svfSaveg, svfEaveg, svfWaveg | canopy blocking buildings |

If the vegetation scheme is off, all veg SVFs default to ones (no blocking).

Meteorological input = the 24-column file

Space-delimited text, 1 header row + 24 columns (UMEP "Prepare Existing Data" format). SOLWEIG reads a subset; the rest may be -999.

| Col (0-idx) | Name | Used? | Variable | Unit |
| --- | --- | --- | --- | --- |
| 0 | iy | ✅ | Year | YYYY |
| 1 | id | ✅ | Day of year (DOY) | 1–366 |
| 2 | it | ✅ | Hour | 0–23 |
| 3 | imin | ✅ | Minute | 0–59 |
| 4 | qn | — | Net radiation | W/m² |
| 5 | qh | — | Sensible heat | W/m² |
| 6 | qe | — | Latent heat | W/m² |
| 7 | qs | — | Storage heat | W/m² |
| 8 | qf | — | Anthropogenic heat | W/m² |
| 9 | U | ✅ | Wind speed Ws | m/s |
| 10 | RH | ✅ | Relative humidity | % |
| 11 | Tair | ✅ | Air temperature Ta | °C |
| 12 | pres | ✅ | Pressure P | kPa |
| 13 | rain | — | Rainfall | mm |
| 14 | kdown | ✅ | Global radiation radG | W/m² |
| 15 | snow | — | Snow | mm |
| 16 | ldown | — | Incoming longwave | W/m² |
| 17 | fcld | — | Cloud fraction | — |
| 18 | wuh | — | External water use | — |
| 19 | xsmd | — | Soil moisture | — |
| 20 | lai | — | Leaf area index | — |
| 21 | kdiff | ✅ | Diffuse radiation radD | W/m² |
| 22 | kdir | ✅ | Direct radiation radI | W/m² |
| 23 | wdir | — | Wind direction | ° |

radD/radI may be -999 only if "Estimate diffuse & direct from global" is ticked.

Ws required only when POIs are used.

A single timestep can be typed in the GUI instead of a file (builds a 1-row version of this array).

Key findings - OUTPUTS (full spec)

### Raster outputs (one GeoTIFF per timestep per checked variable)

### Filename pattern: Var_YYYY_DOY_HHMM[D|N].tif (D = daytime, N = nighttime)

| Output raster | Meaning | Unit | GUI toggle | Day/Night |
| --- | --- | --- | --- | --- |
| Tmrt_* | Mean Radiant Temperature (headline) | °C | CheckBoxTmrt | both |
| Kdown_* | Incoming shortwave | W/m² | CheckBoxKdown | day only |
| Kup_* | Reflected shortwave (upward) | W/m² | CheckBoxKup | day only |
| Ldown_* | Incoming longwave | W/m² | CheckBoxLdown | both |
| Lup_* | Outgoing longwave | W/m² | CheckBoxLup | both |
| Shadow_* | Shadow map (0 shaded → 1 sunlit) | 0–1 | CheckBoxShadow | day only |
| Kdiff* | Diffuse shortwave (TreePlanter/Spatial TC) | W/m² | TreePlanter box | day only |

Aggregate / auxiliary

| File | Meaning | Condition |
| --- | --- | --- |
| Tmrt_average.tif | Run-average Tmrt; auto-styled (tmrt.qml), loaded to canvas | "Add to canvas" |
| buildings.tif | Derived building mask | optional |
| TDSM.tif | Derived trunk-zone DSM | optional |
| RunInfoSOLWEIG_*.txt | Full settings record (reproducibility) | always |

### POI output - the 35-column dictionary

If a POI point layer is selected, each point gets POI_<name>.txt, one row per timestep:

| Col | Name | Meaning | Unit |
| --- | --- | --- | --- |
| 0 | yyyy | Year | YYYY |
| 1 | id | Day of year | DOY |
| 2 | it | Hour | h |
| 3 | imin | Minute | min |
| 4 | dectime | Decimal time | — |
| 5 | altitude | Sun altitude | ° |
| 6 | azimuth | Sun azimuth | ° |
| 7 | kdir | Direct shortwave (radI) | W/m² |
| 8 | kdiff | Diffuse shortwave (radD) | W/m² |
| 9 | kglobal | Global shortwave (radG) | W/m² |
| 10 | kdown | Incoming shortwave at point | W/m² |
| 11 | kup | Reflected shortwave | W/m² |
| 12–15 | keast, ksouth, kwest, knorth | Shortwave from 4 sides | W/m² |
| 16 | ldown | Incoming longwave | W/m² |
| 17 | lup | Outgoing longwave | W/m² |
| 18–21 | least, lsouth, lwest, lnorth | Longwave from 4 sides | W/m² |
| 22 | Ta | Air temperature | °C |
| 23 | Tg | Ground surface temperature | °C |
| 24 | RH | Relative humidity | % |
| 25 | Esky | Sky emissivity | — |
| 26 | Tmrt | Mean Radiant Temperature | °C |
| 27 | I0 | Extraterrestrial radiation | W/m² |
| 28 | CI | Clearness index | — |
| 29 | Shadow | Shadow flag at point | 0–1 |
| 30 | SVF_b | Sky View Factor (buildings) | — |
| 31 | SVF_bv | Sky View Factor (buildings+veg) | — |
| 32 | KsideI | Direct shortwave on cylinder side | W/m² |
| 33 | PET | Physiological Equivalent Temperature | °C |
| 34 | UTCI | Universal Thermal Climate Index | °C |

Wind is rescaled by a power law before comfort indices: 1.1 m for PET, 10 m for UTCI.

### Input → output influence matrix

✅ = strong/direct effect.

| Input / parameter | Tmrt | Kdown | Kup | Ldown | Lup | Shadow | PET/UTCI |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DSM geometry | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ |
| CDSM / TDSM (vegetation) | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ | ✅ |
| SVF set | ✅ | ✅ | ✅ | ✅ | ✅ |  | ✅ |
| Wall height / aspect | ✅ |  |  | ✅ | ✅ | ✅ | ✅ |
| Land cover (albedo/emis) | ✅ | ✅ | ✅ | ✅ | ✅ |  | ✅ |
| radG / radI / radD | ✅ | ✅ | ✅ |  |  |  | ✅ |
| Ta (air temp) | ✅ |  |  | ✅ | ✅ |  | ✅ |
| RH | ✅ |  |  | ✅ |  |  | ✅ |
| Sun position (DOY/UTC/lat) | ✅ | ✅ | ✅ |  |  | ✅ | ✅ |
| absK / absL | ✅ |  |  |  |  |  | ✅ |
| albedo_b (walls) | ✅ | ✅ | ✅ |  |  |  | ✅ |
| ewall (emissivity) | ✅ |  |  | ✅ | ✅ |  | ✅ |
| Posture (standing/sitting) | ✅ |  |  |  |  |  | ✅ |
| Body model (cube/cylinder) | ✅ |  |  |  |  |  | ✅ |
| Sky model (iso/anisotropic) | ✅ | ✅ |  | ✅ |  |  | ✅ |
| Ws (wind speed) |  |  |  |  |  |  | ✅ |

## Parameters & default values

### GUI parameter defaults

| Parameter | Default | Range | Used in |
| --- | --- | --- | --- |
| Global radiation (manual) | 895 | 1–1300 | single-timestep mode |
| Air temperature Ta (manual) | 23 °C | -40–50 | single-timestep mode |
| Relative humidity RH (manual) | 30 % | 1–100 | single-timestep mode |
| Direct radiation radI (manual) | 810 | 1–1200 | single-timestep mode |
| Diffuse radiation radD (manual) | 92.5 | 1–600 | single-timestep mode |
| UTC offset | 1 | -12–12 | sun position |
| Water temperature | 15 °C | -40–50 | water surfaces |
| Wind speed Ws | 3.0 m/s | 0.1–60 | PET/UTCI |
| Sensor height | 10.0 m | ≥0.1 | wind power-law rescale |
| Vegetation transmissivity trans | 3 % |  | shortwave through canopy |
| Albedo, walls albedo_b | 0.20 | 0.01–1 | reflections |
| Albedo, ground albedo_g | 0.15 | 0.01–1 | only if LC scheme off |
| Emissivity, walls ewall | 0.90 | 0.01–1 | longwave |
| Emissivity, ground eground | 0.95 | 0.01–1 | only if LC scheme off |
| Shortwave absorption absK | 0.70 | 0.01–1 | Tmrt |
| Longwave absorption absL | 0.95 | 0.01–1 | Tmrt |
| PET body weight | 75 kg |  | PET |
| PET height | 180 cm |  | PET |

### Hard-coded defaults

| Quantity | Default | Notes |
| --- | --- | --- |
| Altitude (site elevation) | median(DSM), else 3 m | for sun calc |
| Posture Standing | Fside 0.22, Fup 0.06, Fcyl 0.28, h 1.1 m | person↔surface angular factors |
| Posture Sitting | Fside Fup 0.1667, Fcyl 0.2, h 0.75 m |  |
| Stefan–Boltzmann SBC | 5.67051e-8 |  |
| Veg psi (leaf-off) | 0.5 | transmissivity, no leaves |
| Leaf-on / off DOY | 97 / 300 | overridden by GUI for deciduous |
| Surface temp (no LC) | TgK 0.37, Tstart −3.41, TmaxLST 15.0 | cobble-stone-like |
| Clearness index CI | init 1.0, clamped ≤1 | cloudiness |
| Sky model default | Isotropic | anisotropic only if Perez ticked |
| Body model default | Cube | cylinder only if ticked |

### Land-cover property table (landcoverclasses_2016a.txt)

| Name | Code | Albedo | Emis | Ts_deg | Tstart | TmaxLST |
| --- | --- | --- | --- | --- | --- | --- |
| Roofs (buildings) | 2 | 0.18 | 0.95 | 0.58 | -9.78 | 15.0 |
| Dark asphalt | 1 | 0.18 | 0.95 | 0.58 | -9.78 | 15.0 |
| Cobble stone | 0 | 0.20 | 0.95 | 0.37 | -3.41 | 15.0 |
| Water | 7 | 0.05 | 0.98 | 0.00 | 0.00 | 12.0 |
| Grass (unmanaged) | 5 | 0.16 | 0.94 | 0.21 | -3.38 | 14.0 |
| Bare soil | 6 | 0.25 | 0.94 | 0.33 | -3.01 | 14.0 |
| Walls | 99 | 0.20 | 0.90 | 0.58 | -3.41 | 15.0 |

Execution & architecture

Data flow (inputs → model → outputs)

Runtime steps (narrative)

- Load (QGIS startup).

QGIS reads metadata.txt,

calls classFactory in __init__.py → builds UMEP;

UMEP.py adds the menu item.

- Open dialog.

- Collect & validate inputs (start_progress):

read DSM → scale = 1/pixel-size, lat/lon (corner reprojected to WGS84), altitude = median(DSM) (else 3 m)

NoData handling

optional CDSM/TDSM/LC/DEM/walls

extent/resolution check vs DSM

unzip svfs.zip (+ shadowmats.npz if anisotropic, patch count 145/153/306/612).

- Met & solar geometry.

Parse 24-col file Solweig_2015a_metdata_noload → sun position, DOY, daily max altitude.

- Surface-temp maps. LC on → Tgmaps_v1; else uniform defaults.

- Logging. WriteMetadataSOLWEIG.writeRunInfo → settings file.

- Threaded time loop (solweigworker.py): per times